Summary of Requested Services

Soils Report Structural Design Criteria

The report shall be complete and comprehensive, relating all factors influencing the design, construction, and performance of building foundations, superstructure and site improvements.  The report shall include recommendations for the foundation system and alternate systems if appropriate, mediating soil stability, soil properties, water related problems, etc.

The report shall satisfy the requirements of all governing agencies and the corresponding cities/counties where the site is located and specific requirements as required by the District.  The applicable items from the following list should be provided.

Borings:		Establish site reference elevations in USGS. Make relative to design ALTA Survey.

Borings to be a minimum 30’ length or to bedrock bearing strata.

Establish top of bedrock elevation and determine vertical extent of the existing fill in the test hole (s), if any.

Conduct sufficient laboratory testing of retrieved earth material samples for use in geotechnical recommendations preparation.

Provide Soil Types & Classifications/ Gradation/Sieve analysis.

Provide boring logs summarizing the soil conditions and results.

Determine presence of alkali, sulfate or other deleterious material in sufficient quantities to affect concrete or steel including corrosivity analysis.

Address expansive, compressive, weak, or collapsible soils.

Perform soil percolation tests  in area(s) of proposed drainage pond locations.

Groundwater:		Provide depth to water table, establish approximate depth and comment regarding potential seasonal variation.

Determine requirements, if any, for a perimeter drain system.  Provide recommendations.

Determine requirements, if any, for an underslab drain system. Provide recommendations.

Determine if a minimum w/c ratio is required for permeability in foundation concrete.

Footings:		Provide allowable bearing pressure and minimum dead load pressure.

Establish appropriate bearing strata and identify depth.

Provide extent of soil overexcavation and recompaction under footings.

Determine requirements of imported fill, if any.

Determine possibility and estimate of differential settlement.

Provide frost depth.

Drilled Piers:		Determine allowable end bearing and side shear.

Determine if upper bedrock is weathered.  If so, comment on weathered thickness.

Determine if penetration can be neglected for side shear.

Determine if side shear should be reduced for uplift and if the uplift penetration is in addition to minimum.

Determine if shear rings or roughening is required.

Determine minimum penetration into bedrock and minimum pier length.

Determine minimum pier diameter and pier spacing requirements.

Determine void form thickness.

Determine possibility and estimate of differential settlement.

Determine if casing will be required.

Provide concrete requirements: slump, w/c ratio.

Provide L-Pile In-Put Data.

Slab-on-Grade:	Determine if a gravel layer and/or vapor barrier is recommended.

Determine extent of over-excavation and re-compaction, if any.

Determine requirements of imported fill, if any.  Provide import fill material requirements.

If swelling soils exist, provide criteria for risk assessment including estimate of slab movements with alternative underslab treatments.

Provide modulus of Subgrade Reaction.

Provide possibility and estimate of differential settlement or heave.

Discuss control joint spacing and slab separation.

Determine if isolation joints required at the top of partitions.

Retaining Walls:	  	Provide Equivalent Fluid Pressures for restrained and unrestrained conditions.

Determine backfill requirements.

Determine passive pressures and sliding coefficients.

Cement Type:		Specify special requirements particular to site.

Provide sulfate resistant cement requirements.

Parking Lot:	Provide sub-base and paving recommendations for accesses and parking lots.

Provide min. slope recommendations.

Underground Utilites:Provide pipe bedding & compaction recommendations.

Provide corrosion considerations.

General:		IBC Soil Classification for Seismic Analysis.

Determine foundation criteria other than footings or drilled piers as required.

Provide complete and adequate borings in the proposed parking lot, service and drive areas to provide design details for site flatwork (asphalt and concrete for walks, drives, parking areas, loading/truck areas) building, and other site improvement areas.  Include pavement section details with subgrade preparation recommendations.

If it does not appear appropriate to require some of the above services, please bring such items to our attention.  However, include the cost of any such items in your base response.

If there are additional services which are not mentioned above that your firm feels would be necessary or advantageous please bring those services to our attention and provide a breakout, line item cost for each additional service.

Additional Services

To be Quoted:		Provide hourly rates for all levels of office and field personnel.

Estimated reimbursables for the project are to be listed separately from the base proposal and line items are to be provided for each reimbursable type.

Review plans and specifications during the design stage to see if recommendations have been properly interpreted.

Respond/Address comments during plan check review process on comments regarding soils report items.

Once excavation has commenced, conduct site visits to ascertain whether conditions actually encountered are the same as those anticipated in investigation/soils report, and whether recommendations have been properly implemented.